Test System Data for “5G-Enhanced Load Restoration for Interdependent Distribution-Cyber-Transportation Systems with Communication-Constrained V2G Responses”

Test System 1: IEEE benchmark system

- Network Topology：

(a) DN topology                           (b) CN topology                           (c) TN topology

Fig. 1 Topologies of the IEEE benchmark system

- Detailed Parameters：

Table 1. DN line parameters

| Line | From_bus | To_bus | Type | R/pu | X/pu | S/MVA | Line | From_bus | To_bus | Type | R/pu | X/pu | S/MVA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 | 1 | 2 | Ordinary | 0.005753 | 0.002932 | 5.33 | D20 | 20 | 21 | Ordinary | 0.025550 | 0.029849 | 0.80 |
| D2 | 2 | 3 | Ordinary | 0.030760 | 0.015667 | 5.00 | D21 | 21 | 22 | Ordinary | 0.044230 | 0.058481 | 0.25 |
| D3 | 3 | 4 | Ordinary | 0.022836 | 0.011630 | 4.00 | D22 | 3 | 23 | Ordinary | 0.028152 | 0.019236 | 1.60 |
| D4 | 4 | 5 | Ordinary | 0.023778 | 0.012110 | 3.20 | D23 | 23 | 24 | Ordinary | 0.056028 | 0.044243 | 1.60 |
| D5 | 5 | 6 | Ordinary | 0.051099 | 0.044112 | 3.15 | D24 | 24 | 25 | Ordinary | 0.055904 | 0.043743 | 0.80 |
| D6 | 6 | 7 | Ordinary | 0.011680 | 0.038608 | 2.00 | D25 | 6 | 26 | Ordinary | 0.012666 | 0.006451 | 3.15 |
| D7 | 7 | 8 | Ordinary | 0.044386 | 0.014668 | 1.60 | D26 | 26 | 27 | Ordinary | 0.017732 | 0.009028 | 3.15 |
| D8 | 8 | 9 | Ordinary | 0.064264 | 0.046170 | 1.25 | D27 | 27 | 28 | Ordinary | 0.066074 | 0.058256 | 2.34 |
| D9 | 9 | 10 | Ordinary | 0.065138 | 0.046170 | 1.25 | D28 | 28 | 29 | Ordinary | 0.050176 | 0.043712 | 3.15 |
| D10 | 10 | 11 | Ordinary | 0.012266 | 0.004056 | 1.00 | D29 | 29 | 30 | Ordinary | 0.031664 | 0.016128 | 1.10 |
| D11 | 11 | 12 | Ordinary | 0.023360 | 0.007724 | 1.00 | D30 | 30 | 31 | Ordinary | 0.060795 | 0.060084 | 0.80 |
| D12 | 12 | 13 | Ordinary | 0.091592 | 0.072063 | 0.80 | D31 | 31 | 32 | Ordinary | 0.019373 | 0.022580 | 0.50 |
| D13 | 13 | 14 | Ordinary | 0.033792 | 0.044480 | 0.80 | D32 | 32 | 33 | Ordinary | 0.021276 | 0.033081 | 0.25 |
| D14 | 14 | 15 | Ordinary | 0.036874 | 0.032818 | 0.50 | D33 | 21 | 8 | Tie | 0.124785 | 0.124785 | 0.40 |
| D15 | 15 | 16 | Ordinary | 0.046564 | 0.034004 | 0.32 | D34 | 9 | 15 | Tie | 0.124785 | 0.124785 | 0.50 |
| D16 | 16 | 17 | Ordinary | 0.080424 | 0.107378 | 0.40 | D35 | 12 | 22 | Tie | 0.124785 | 0.124785 | 0.25 |
| D17 | 17 | 18 | Ordinary | 0.045671 | 0.035813 | 0.50 | D36 | 18 | 33 | Tie | 0.031196 | 0.031196 | 0.50 |
| D18 | 2 | 19 | Ordinary | 0.010232 | 0.009764 | 1.00 | D37 | 25 | 29 | Tie | 0.031196 | 0.031196 | 1.20 |
| D19 | 19 | 20 | Ordinary | 0.093851 | 0.084567 | 1.00 | - | - | - | - | - | - | - |

Table 2. DN bus and distributed resource parameters

| Bus | Type | PL/MW | QL/Mvar | P/MW | Q/Mvar | S/MVA | Bus | Type | PL/MW | QL/Mvar | P/MW | Q/Mvar | S/MVA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Substation | 0.000 | 0.000 | 4.000 | 2.600 | - | 18 | Load, PV | 0.126 | 0.056 | 0.200 | 0.100 | 0.224 |
| 2 | Load | 0.140 | 0.084 | 0.000 | 0.000 | 0.000 | 19 | Load | 0.126 | 0.056 | 0.000 | 0.000 | 0.000 |
| 3 | Load | 0.126 | 0.056 | 0.000 | 0.000 | 0.000 | 20 | Load | 0.126 | 0.056 | 0.000 | 0.000 | 0.000 |
| 4 | Load | 0.168 | 0.112 | 0.000 | 0.000 | 0.000 | 21 | Load, MT | 0.126 | 0.056 | 0.150 | 0.150 | 0.000 |
| 5 | Load | 0.084 | 0.042 | 0.000 | 0.000 | 0.000 | 22 | Load | 0.126 | 0.056 | 0.000 | 0.000 | 0.000 |
| 6 | Load, MT | 0.084 | 0.028 | 0.350 | 0.350 | 0.000 | 23 | Load | 0.126 | 0.070 | 0.000 | 0.000 | 0.000 |
| 7 | Load, PV | 0.280 | 0.140 | 0.300 | 0.150 | 0.336 | 24 | Load | 0.588 | 0.280 | 0.000 | 0.000 | 0.000 |
| 8 | Load | 0.280 | 0.140 | 0.000 | 0.000 | 0.000 | 25 | Load | 0.588 | 0.280 | 0.000 | 0.000 | 0.000 |
| 9 | Load | 0.084 | 0.028 | 0.000 | 0.000 | 0.000 | 26 | Load | 0.084 | 0.035 | 0.000 | 0.000 | 0.000 |
| 10 | Load | 0.084 | 0.028 | 0.000 | 0.000 | 0.000 | 27 | Load | 0.084 | 0.035 | 0.000 | 0.000 | 0.000 |
| 11 | Load | 0.063 | 0.042 | 0.000 | 0.000 | 0.000 | 28 | Load, PV | 0.084 | 0.028 | 0.150 | 0.075 | 0.168 |
| 12 | Load | 0.084 | 0.049 | 0.000 | 0.000 | 0.000 | 29 | Load | 0.168 | 0.098 | 0.000 | 0.000 | 0.000 |
| 13 | Load | 0.084 | 0.049 | 0.000 | 0.000 | 0.000 | 30 | Load | 0.280 | 0.840 | 0.000 | 0.000 | 0.000 |
| 14 | Load | 0.168 | 0.112 | 0.000 | 0.000 | 0.000 | 31 | Load | 0.210 | 0.098 | 0.000 | 0.000 | 0.000 |
| 15 | Load | 0.084 | 0.014 | 0.000 | 0.000 | 0.000 | 32 | Load | 0.294 | 0.140 | 0.000 | 0.000 | 0.000 |
| 16 | Load | 0.084 | 0.028 | 0.000 | 0.000 | 0.000 | 33 | Load, PV | 0.084 | 0.056 | 0.150 | 0.075 | 0.168 |
| 17 | Load | 0.084 | 0.028 | 0.000 | 0.000 | 0.000 | - | - | - | - | - | - | - |

Table 3. CN communication link parameters

| Link | From | To | Type | Capacity | Delay | Link | From | To | Type | Capacity | Delay | Link | From | To | Type | Capacity | Delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 | Server | RT1 | Wired | 100 | 1 | C23 | SW7 | FTU12 | Wired | 50 | 1 | C45 | SW5 | FTU34 | Wired | 50 | 1 |
| C2 | Server | RT2 | Wired | 100 | 1 | C24 | SW7 | FTU13 | Wired | 50 | 1 | C46 | SW7 | FTU35 | Wired | 50 | 1 |
| C3 | RT1 | RT2 | Wired | 100 | 1 | C25 | SW8 | FTU14 | Wired | 50 | 1 | C47 | SW5 | FTU36 | Wired | 50 | 1 |
| C4 | RT1 | SW1 | Wired | 100 | 1 | C26 | SW8 | FTU15 | Wired | 50 | 1 | C48 | SW8 | FTU37 | Wired | 50 | 1 |
| C5 | SW1 | SW5 | Wired | 100 | 1 | C27 | SW8 | FTU16 | Wired | 50 | 1 | C49 | SW3 | FTU38 | Wired | 50 | 1 |
| C6 | SW7 | SW8 | Wired | 100 | 1 | C28 | SW8 | FTU17 | Wired | 50 | 1 | C50 | BS1 | Server | 5G | 200 | 5 |
| C7 | RT2 | SW2 | Wired | 100 | 1 | C29 | SW8 | FTU18 | Wired | 50 | 1 | C51 | BS2 | Server | 5G | 200 | 5 |
| C8 | SW2 | SW3 | Wired | 100 | 1 | C30 | SW1 | FTU19 | Wired | 50 | 1 | C52 | BS3 | Server | 5G | 200 | 5 |
| C9 | SW3 | SW6 | Wired | 100 | 1 | C31 | SW1 | FTU20 | Wired | 50 | 1 | C53 | BS1 | FTU4 | 5G | 200 | 5 |
| C10 | SW6 | SW8 | Wired | 100 | 1 | C32 | SW5 | FTU21 | Wired | 50 | 1 | C54 | BS1 | FTU5 | 5G | 200 | 5 |
| C11 | SW3 | SW4 | Wired | 100 | 1 | C33 | SW5 | FTU22 | Wired | 50 | 1 | C55 | BS1 | FTU6 | 5G | 200 | 5 |
| C12 | SW6 | SW7 | Wired | 100 | 1 | C34 | SW2 | FTU23 | Wired | 50 | 1 | C56 | BS1 | FTU24 | 5G | 200 | 5 |
| C13 | SW1 | FTU2 | Wired | 50 | 1 | C35 | SW2 | FTU24 | Wired | 50 | 1 | C57 | BS1 | FTU25 | 5G | 200 | 5 |
| C14 | SW5 | FTU3 | Wired | 50 | 1 | C36 | SW2 | FTU25 | Wired | 50 | 1 | C58 | BS1 | FTU26 | 5G | 200 | 5 |
| C15 | SW5 | FTU4 | Wired | 50 | 1 | C37 | SW3 | FTU26 | Wired | 50 | 1 | C59 | BS2 | FTU9 | 5G | 200 | 5 |
| C16 | SW5 | FTU5 | Wired | 50 | 1 | C38 | SW3 | FTU27 | Wired | 50 | 1 | C60 | BS2 | FTU10 | 5G | 200 | 5 |
| C17 | SW4 | FTU6 | Wired | 50 | 1 | C39 | SW3 | FTU28 | Wired | 50 | 1 | C61 | BS2 | FTU11 | 5G | 200 | 5 |
| C18 | SW4 | FTU7 | Wired | 50 | 1 | C40 | SW3 | FTU29 | Wired | 50 | 1 | C62 | BS2 | FTU36 | 5G | 200 | 5 |
| C19 | SW4 | FTU8 | Wired | 50 | 1 | C41 | SW6 | FTU30 | Wired | 50 | 1 | C63 | BS3 | FTU14 | 5G | 200 | 5 |
| C20 | SW4 | FTU9 | Wired | 50 | 1 | C42 | SW6 | FTU31 | Wired | 50 | 1 | C64 | BS3 | FTU15 | 5G | 200 | 5 |
| C21 | SW7 | FTU10 | Wired | 50 | 1 | C43 | SW6 | FTU32 | Wired | 50 | 1 | C65 | BS3 | FTU33 | 5G | 200 | 5 |
| C22 | SW7 | FTU11 | Wired | 50 | 1 | C44 | SW6 | FTU33 | Wired | 50 | 1 | C66 | BS3 | FTU37 | 5G | 200 | 5 |

Table 4. TN road parameters

| Road | From | To | Capacity/(veh·h⁻¹) | Length/m | Free-flow time/h | Road | From | To | Capacity/(veh·h⁻¹) | Length/m | Free-flow time/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | T1 | T2 | 200 | 719 | 0.05033 | R20 | T5 | T9 | 150 | 900 | 0.06300 |
| R2 | T2 | T3 | 300 | 1200 | 0.08400 | R21 | T9 | T13 | 120 | 550 | 0.03850 |
| R3 | T3 | T4 | 300 | 786 | 0.05502 | R22 | T13 | T17 | 650 | 3155 | 0.22085 |
| R4 | T5 | T6 | 180 | 814 | 0.05698 | R23 | T2 | T6 | 300 | 714 | 0.04998 |
| R5 | T6 | T7 | 200 | 1200 | 0.08400 | R24 | T6 | T10 | 300 | 728 | 0.05096 |
| R6 | T7 | T8 | 100 | 971 | 0.06797 | R25 | T10 | T14 | 260 | 548 | 0.03836 |
| R7 | T9 | T10 | 200 | 883 | 0.06181 | R26 | T14 | T18 | 270 | 646 | 0.04522 |
| R8 | T10 | T11 | 200 | 1100 | 0.07700 | R27 | T18 | T21 | 265 | 619 | 0.04333 |
| R9 | T11 | T12 | 250 | 946 | 0.06622 | R28 | T4 | T7 | 214.5 | 1300 | 0.09100 |
| R10 | T13 | T14 | 180 | 899 | 0.06293 | R29 | T7 | T11 | 225 | 685 | 0.04795 |
| R11 | T14 | T15 | 180 | 962 | 0.06734 | R30 | T11 | T15 | 400 | 1142 | 0.07994 |
| R12 | T15 | T16 | 175 | 950 | 0.06650 | R31 | T15 | T19 | 440 | 1314 | 0.09198 |
| R13 | T17 | T18 | 220 | 950 | 0.06650 | R32 | T19 | T22 | 400 | 1102.5 | 0.077175 |
| R14 | T18 | T19 | 220 | 826 | 0.05782 | R33 | T4 | T8 | 215 | 1200 | 0.08400 |
| R15 | T19 | T20 | 230 | 955 | 0.06685 | R34 | T8 | T12 | 280 | 687 | 0.04809 |
| R16 | T17 | T21 | 370 | 2300 | 0.16100 | R35 | T12 | T16 | 575 | 1427.5 | 0.099925 |
| R17 | T21 | T22 | 130 | 758 | 0.05306 | R36 | T16 | T20 | 270 | 646 | 0.04522 |
| R18 | T22 | T23 | 217.5 | 1468.5 | 0.102795 | R37 | T20 | T23 | 225 | 550 | 0.03850 |
| R19 | T1 | T5 | 240 | 1136 | 0.07952 | - | - | - | - | - | - |

Table 5. Origin-destination pairs and candidate paths

| O-D pair | O | D | Trip type | O-D pair | O | D | Trip type | O-D pair | O | D | Trip type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T10 | T1 | To work | 7 | T1 | T22 | To leisure | 13 | T17 | T12 | From work |
| 2 | T3 | T17 | To work | 8 | T12 | T2 | To leisure | 14 | T22 | T12 | From work |
| 3 | T3 | T22 | To work | 9 | T23 | T15 | To leisure | 15 | T2 | T1 | From leisure |
| 4 | T12 | T17 | To work | 10 | T1 | T10 | From work | 16 | T22 | T1 | From leisure |
| 5 | T12 | T22 | To work | 11 | T17 | T3 | From work | 17 | T2 | T12 | From leisure |
| 6 | T1 | T2 | To leisure | 12 | T22 | T3 | From work | 18 | T15 | T23 | From leisure |

Table 6. Hourly origin-destination travel demand

| OD pair Period | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 40 | 0 | 25 | 0 | 0 | 20 | 0 | 10 | 0 | 0 | 0 | 5 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 25 | 35 | 80 | 50 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | 0 | 0 | 0 | 20 | 35 | 55 | 80 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 15 | 30 | 75 | 60 | 15 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 15 | 35 | 65 | 55 | 25 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 15 | 5 | 0 | 0 | 3 | 10 | 12 | 5 | 3 | 2 | 0 | 0 | 0 | 0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 3 | 2 | 6 | 8 | 12 | 3 | 1 | 2 | 3 | 5 | 10 | 5 | 3 | 2 | 0 | 0 | 0 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 8 | 10 | 15 | 9 | 6 | 7 | 7 | 14 | 8 | 10 | 3 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 11 | 18 | 8 | 6 | 5 | 2 | 7 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 15 | 0 | 0 | 20 | 0 | 0 | 0 | 0 | 0 | 40 | 0 | 0 | 25 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 15 | 25 | 50 | 35 | 25 | 30 | 15 | 5 | 0 |
| 12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 25 | 55 | 35 | 15 | 40 | 15 | 3 | 2 |
| 13 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 30 | 40 | 50 | 35 | 20 | 10 | 5 | 3 |
| 14 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 25 | 55 | 30 | 20 | 30 | 15 | 7 | 5 |
| 15 | 2 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 5 | 8 | 6 | 3 | 4 | 0 | 2 | 5 | 9 | 10 | 5 | 3 |
| 16 | 4 | 3 | 4 | 3 | 3 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 5 | 4 | 2 | 1 | 0 | 6 | 7 | 5 | 4 | 9 | 5 | 3 |
| 17 | 3 | 2 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 8 | 10 | 9 | 6 | 7 | 2 | 5 | 6 | 10 | 13 | 10 | 6 |
| 18 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 5 | 7 | 5 | 3 | 10 | 9 | 3 | 6 | 7 | 6 | 1 |

Table 7. Multi-fault combinations

| Fault combination | DN line faults | CN link faults | Fault combination | DN line faults | CN link faults |
| --- | --- | --- | --- | --- | --- |
| L01 | D7, D9, D33 | C9, C47 | M06 | D4, D7, D11, D19, D25, D28, D34 | C9, C11, C15, C23, C38, C43 |
| L02 | D6, D12, D37 | C9, C42 | M07 | D5, D7, D12, D15, D18, D21, D34 | C3, C4, C9, C19, C23, C42 |
| L03 | D7, D10, D33 | C9, C47 | M08 | D6, D8, D10, D13, D30 | C3, C9, C16, C42 |
| L04 | D8, D9, D33 | C9, C47 | M09 | D4, D6, D8, D9, D16, D28, D34 | C5, C10, C11, C16, C19, C49 |
| L05 | D19, D24, D32 | C11, C46 | M10 | D5, D7, D22 | C5, C23 |
| L06 | D7, D9, D32 | C9, C47 | S01 | D6, D21, D27, D29, D31 | C8, C11, C29, C49 |
| L07 | D12, D21, D25 | C5, C31 | S02 | D6, D21, D27, D29, D32 | C8, C11, C29, C49 |
| L08 | D9, D11, D12 | C7, C34 | S03 | D8, D15, D20, D24, D28 | C6, C8, C43, C44 |
| L09 | D13, D19, D21, D23, D31 | C9, C11, C33, C41 | S04 | D4, D8, D22, D24, D25, D28, D31 | C4, C7, C12, C21, C36, C46 |
| L10 | D11, D19, D20, D30, D37 | C7, C17, C21, C28 | S05 | D16, D18, D21, D22, D28, D32, D34 | C4, C5, C6, C14, C16, C20 |
| M01 | D5, D9, D21, D27, D36 | C4, C21, C25, C48 | S06 | D15, D17, D19, D23, D24, D25, D26 | C6, C9, C11, C18, C31, C45 |
| M02 | D7, D12, D21, D23, D32 | C5, C9, C17, C44 | S07 | D8, D11, D16, D18, D22, D27, D30 | C3, C9, C10, C26, C35, C41 |
| M03 | D10, D17, D22, D24, D36 | C8, C9, C26, C29 | S08 | D13, D14, D18, D20, D23, D26, D33 | C5, C6, C9, C32, C45, C46 |
| M04 | D13, D17, D29 | C8, C40 | S09 | D4, D6, D13, D23, D29, D35, D36 | C5, C6, C11, C35, C42, C45 |
| M05 | D9, D21, D28, D32, D37 | C8, C11, C18, C22 | S10 | D5, D10, D19, D23, D28, D30, D35 | C4, C11, C27, C29, C36, C47 |

Note: L, M, and S denote light, moderate, and severe fault combinations, respectively.

Test System 2: large-scale real-world system

- Network Topology：

(a) DN topology

(b) CN topology

(c) TN topology

Fig. 2 Topologies of the large-scale real-world system

- Detailed Parameters：

Table 8. DN line parameters

| Line | From_bus | To_bus | Type | R/pu | X/pu | S/MVA | Line | From_bus | To_bus | Type | R/pu | X/pu | S/MVA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 | 1 | 2 | Ordinary | 0.002975 | 0.001071 | 16.26 | D67 | 67 | 68 | Ordinary | 0.041653 | 0.027298 | 3.75 |
| D2 | 2 | 3 | Ordinary | 0.002727 | 0.000982 | 0.3 | D68 | 68 | 69 | Ordinary | 0.033554 | 0.012074 | 3.68 |
| D3 | 2 | 4 | Ordinary | 0.003719 | 0.001339 | 12.90 | D69 | 69 | 70 | Ordinary | 0.079504 | 0.062893 | 3.58 |
| D4 | 4 | 5 | Ordinary | 0.00124 | 0.004463 | 0.63 | D70 | 70 | 71 | Ordinary | 0.013636 | 0.004959 | 2.82 |
| D5 | 5 | 6 | Ordinary | 0.00124 | 0.004463 | 0.52 | D71 | 71 | 72 | Ordinary | 0.025041 | 0.009025 | 2.04 |
| D6 | 6 | 7 | Ordinary | 0.00124 | 0.001033 | 0.32 | D72 | 72 | 73 | Ordinary | 0.025041 | 0.009025 | 1.84 |
| D7 | 7 | 8 | Ordinary | 0.001488 | 0.001157 | 0.30 | D73 | 73 | 74 | Ordinary | 0.017025 | 0.011901 | 1.77 |
| D8 | 8 | 9 | Ordinary | 0.001736 | 0.005207 | 0.30 | D74 | 74 | 75 | Ordinary | 0.019256 | 0.006942 | 0.44 |
| D9 | 2 | 10 | Ordinary | 0.013719 | 0.011107 | 3.14 | D75 | 75 | 76 | Ordinary | 0.048843 | 0.014653 | 0.38 |
| D10 | 10 | 11 | Ordinary | 0.009256 | 0.006521 | 2.86 | D76 | 76 | 77 | Ordinary | 0.010413 | 0.003744 | 0.30 |
| D11 | 11 | 12 | Ordinary | 0.015455 | 0.025868 | 0.49 | D77 | 64 | 78 | Ordinary | 0.046198 | 0.030471 | 3.42 |
| D12 | 12 | 13 | Ordinary | 0.011736 | 0.012496 | 0.45 | D78 | 78 | 79 | Ordinary | 0.015372 | 0.01014 | 3.00 |
| D13 | 13 | 14 | Ordinary | 0.014876 | 0.009752 | 0.38 | D79 | 79 | 80 | Ordinary | 0.015372 | 0.01014 | 1.91 |
| D14 | 14 | 15 | Ordinary | 0.012397 | 0.003719 | 0.30 | D80 | 80 | 81 | Ordinary | 0.021488 | 0.011488 | 1.10 |
| D15 | 15 | 16 | Ordinary | 0.013223 | 0.014876 | 0.30 | D81 | 81 | 82 | Ordinary | 0.012727 | 0.012231 | 0.72 |
| D16 | 16 | 17 | Ordinary | 0.012975 | 0.014132 | 0.30 | D82 | 82 | 83 | Ordinary | 0.019008 | 0.010579 | 0.34 |
| D17 | 11 | 18 | Ordinary | 0.018017 | 0.023554 | 2.16 | D83 | 83 | 84 | Ordinary | 0.020826 | 0.00876 | 0.30 |
| D18 | 18 | 19 | Ordinary | 0.009752 | 0.015289 | 2.14 | D84 | 84 | 85 | Ordinary | 0.014876 | 0.012231 | 0.30 |
| D19 | 19 | 20 | Ordinary | 0.013223 | 0.016198 | 1.92 | D85 | 79 | 86 | Ordinary | 0.013223 | 0.015041 | 0.68 |
| D20 | 20 | 21 | Ordinary | 0.009917 | 0.01562 | 1.11 | D86 | 86 | 87 | Ordinary | 0.016529 | 0.019008 | 0.30 |
| D21 | 21 | 22 | Ordinary | 0.009917 | 0.006521 | 0.82 | D87 | 87 | 88 | Ordinary | 0.013223 | 0.032479 | 0.30 |
| D22 | 22 | 23 | Ordinary | 0.116529 | 0.059752 | 0.67 | D88 | 65 | 89 | Ordinary | 0.055289 | 0.019934 | 2.28 |
| D23 | 23 | 24 | Ordinary | 0.024215 | 0.01114 | 0.56 | D89 | 89 | 90 | Ordinary | 0.021983 | 0.01014 | 2.19 |
| D24 | 24 | 25 | Ordinary | 0.010992 | 0.008595 | 0.33 | D90 | 90 | 91 | Ordinary | 0.021983 | 0.01014 | 2.13 |
| D25 | 25 | 26 | Ordinary | 0.014711 | 0.011074 | 0.30 | D91 | 91 | 92 | Ordinary | 0.021983 | 0.01014 | 0.43 |
| D26 | 26 | 27 | Ordinary | 0.014711 | 0.011074 | 0.30 | D92 | 92 | 93 | Ordinary | 0.021983 | 0.01014 | 0.30 |
| D27 | 4 | 28 | Ordinary | 0.00124 | 0.002446 | 12.22 | D93 | 93 | 94 | Ordinary | 0.019256 | 0.009504 | 0.30 |
| D28 | 28 | 29 | Ordinary | 0.000992 | 0.002281 | 11.23 | D94 | 94 | 95 | Ordinary | 0.040992 | 0.011405 | 0.30 |
| D29 | 29 | 30 | Ordinary | 0.009917 | 0.02286 | 7.23 | D95 | 91 | 96 | Ordinary | 0.016198 | 0.014876 | 1.60 |
| D30 | 30 | 31 | Ordinary | 0.017355 | 0.020083 | 5.65 | D96 | 96 | 97 | Ordinary | 0.016198 | 0.014876 | 0.89 |
| D31 | 31 | 32 | Ordinary | 0.009917 | 0.004463 | 4.90 | D97 | 97 | 98 | Ordinary | 0.015421 | 0.010083 | 0.30 |
| D32 | 32 | 33 | Ordinary | 0.014711 | 0.019339 | 4.08 | D98 | 98 | 99 | Ordinary | 0.006165 | 0.026281 | 0.30 |
| D33 | 33 | 34 | Ordinary | 0.014711 | 0.019339 | 3.83 | D99 | 1 | 100 | Ordinary | 0.005165 | 0.00219 | 8.01 |
| D34 | 34 | 35 | Ordinary | 0.012727 | 0.013388 | 3.57 | D100 | 100 | 101 | Ordinary | 0.012405 | 0.019339 | 7.07 |
| D35 | 30 | 36 | Ordinary | 0.015455 | 0.02157 | 1.41 | D101 | 101 | 102 | Ordinary | 0.011132 | 0.007339 | 6.35 |
| D36 | 36 | 37 | Ordinary | 0.010992 | 0.008182 | 0.68 | D102 | 102 | 103 | Ordinary | 0.019066 | 0.009942 | 5.49 |
| D37 | 29 | 38 | Ordinary | 0.027273 | 0.016033 | 2.47 | D103 | 103 | 104 | Ordinary | 0.036942 | 0.013289 | 4.96 |
| D38 | 38 | 39 | Ordinary | 0.02562 | 0.016033 | 2.32 | D104 | 104 | 105 | Ordinary | 0.013488 | 0.00486 | 4.72 |
| D39 | 39 | 40 | Ordinary | 0.010744 | 0.016033 | 2.23 | D105 | 105 | 106 | Ordinary | 0.027273 | 0.008182 | 4.56 |
| D40 | 40 | 41 | Ordinary | 0.02314 | 0.012397 | 1.59 | D106 | 106 | 107 | Ordinary | 0.012893 | 0.004636 | 4.40 |
| D41 | 41 | 42 | Ordinary | 0.097521 | 0.070248 | 1.06 | D107 | 107 | 108 | Ordinary | 0.031562 | 0.011355 | 3.59 |
| D42 | 42 | 43 | Ordinary | 0.034711 | 0.020132 | 0.33 | D108 | 108 | 109 | Ordinary | 0.013438 | 0.004835 | 3.27 |
| D43 | 43 | 44 | Ordinary | 0.022314 | 0.008033 | 0.30 | D109 | 109 | 110 | Ordinary | 0.031562 | 0.011355 | 2.47 |
| D44 | 44 | 45 | Ordinary | 0.028017 | 0.010091 | 0.30 | D110 | 110 | 111 | Ordinary | 0.020207 | 0.007264 | 1.61 |
| D45 | 45 | 46 | Ordinary | 0.022314 | 0.014702 | 0.30 | D111 | 110 | 112 | Ordinary | 0.017256 | 0.006223 | 0.55 |
| D46 | 35 | 47 | Ordinary | 0.017355 | 0.01143 | 3.37 | D112 | 112 | 113 | Ordinary | 0.019017 | 0.006843 | 0.30 |
| D47 | 47 | 48 | Ordinary | 0.009917 | 0.006521 | 3.27 | D113 | 100 | 114 | Ordinary | 0.05043 | 0.018149 | 0.81 |
| D48 | 48 | 49 | Ordinary | 0.012397 | 0.008157 | 3.16 | D114 | 114 | 115 | Ordinary | 0.015421 | 0.010496 | 0.46 |
| D49 | 49 | 50 | Ordinary | 0.012397 | 0.008157 | 3.01 | D115 | 115 | 116 | Ordinary | 0.030843 | 0.020331 | 0.35 |
| D50 | 50 | 51 | Ordinary | 0.019835 | 0.013066 | 1.15 | D116 | 116 | 117 | Ordinary | 0.033471 | 0.030331 | 0.30 |
| D51 | 51 | 52 | Ordinary | 0.009917 | 0.006521 | 0.83 | D117 | 117 | 118 | Ordinary | 0.040413 | 0.036198 | 0.30 |
| D52 | 52 | 53 | Ordinary | 0.033471 | 0.01205 | 0.72 | D118 | 46 | 27 | Tie | 0.043455 | 0.024174 | 1.00 |
| D53 | 53 | 54 | Ordinary | 0.033471 | 0.01205 | 0.65 | D119 | 17 | 27 | Tie | 0.043455 | 0.024099 | 1.00 |
| D54 | 29 | 55 | Ordinary | 0.032314 | 0.011653 | 1.38 | D120 | 8 | 24 | Tie | 0.035306 | 0.012719 | 1.00 |
| D55 | 55 | 56 | Ordinary | 0.033554 | 0.012074 | 1.30 | D121 | 54 | 43 | Tie | 0.039669 | 0.014281 | 1.00 |
| D56 | 56 | 57 | Ordinary | 0.033554 | 0.012074 | 1.14 | D122 | 62 | 49 | Tie | 0.029752 | 0.010711 | 1.73 |
| D57 | 57 | 58 | Ordinary | 0.058347 | 0.045132 | 1.01 | D123 | 37 | 62 | Tie | 0.047107 | 0.047273 | 1.00 |
| D58 | 58 | 59 | Ordinary | 0.027934 | 0.010066 | 0.42 | D124 | 9 | 40 | Tie | 0.043802 | 0.027669 | 1.27 |
| D59 | 59 | 60 | Ordinary | 0.027934 | 0.010066 | 0.38 | D125 | 58 | 96 | Tie | 0.032702 | 0.011777 | 1.31 |
| D60 | 60 | 61 | Ordinary | 0.017107 | 0.006174 | 0.30 | D126 | 73 | 91 | Tie | 0.056198 | 0.053554 | 1.99 |
| D61 | 61 | 62 | Ordinary | 0.020413 | 0.073736 | 0.30 | D127 | 88 | 75 | Tie | 0.03357 | 0.012099 | 1.00 |
| D62 | 1 | 63 | Ordinary | 0.002314 | 0.003455 | 11.22 | D128 | 99 | 77 | Tie | 0.038231 | 0.013835 | 1.00 |
| D63 | 63 | 64 | Ordinary | 0.009669 | 0.016661 | 10.40 | D129 | 108 | 83 | Tie | 0.053802 | 0.019339 | 1.97 |
| D64 | 64 | 65 | Ordinary | 0.021074 | 0.007587 | 6.84 | D130 | 105 | 86 | Tie | 0.067149 | 0.024174 | 2.70 |
| D65 | 65 | 66 | Ordinary | 0.017355 | 0.006273 | 4.34 | D131 | 110 | 118 | Tie | 0.058587 | 0.021099 | 1.39 |
| D66 | 66 | 67 | Ordinary | 0.031653 | 0.011405 | 3.80 | D132 | 25 | 35 | Tie | 0.041322 | 0.041322 | 1.95 |

Table 9. DN bus and distributed resource parameters

| Bus | Type | PL/MW | QL/Mvar | P/MW | Q/Mvar | S/MVA | Bus | Type | PL/MW | QL/Mvar | P/MW | Q/Mvar | S/MVA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Substation | 0 | 0.000 | 25.000 | 20.000 | - | 60 | Load, PV | 0.080551 | 0.049156 | 0.400 | 0.000 | 0.400 |
| 2 | Load | 0.13384 | 0.10114 | 0.000 | 0.000 | 0.000 | 61 | Load | 0.09586 | 0.090758 | 0.000 | 0.000 | 0.000 |
| 3 | Load | 0.016214 | 0.011292 | 0.000 | 0.000 | 0.000 | 62 | Load | 0.06292 | 0.0477 | 0.000 | 0.000 | 0.000 |
| 4 | Load | 0.034315 | 0.021845 | 0.000 | 0.000 | 0.000 | 63 | Load | 0.4788 | 0.46374 | 0.000 | 0.000 | 0.000 |
| 5 | Load | 0.073016 | 0.063602 | 0.000 | 0.000 | 0.000 | 64 | Load | 0.12094 | 0.052006 | 0.000 | 0.000 | 0.000 |
| 6 | Load | 0.1442 | 0.068604 | 0.000 | 0.000 | 0.000 | 65 | Load | 0.13911 | 0.10034 | 0.000 | 0.000 | 0.000 |
| 7 | Load | 0.10447 | 0.061725 | 0.000 | 0.000 | 0.000 | 66 | Load, PV | 0.39178 | 0.1935 | 0.450 | 0.000 | 0.450 |
| 8 | Load | 0.028547 | 0.011503 | 0.000 | 0.000 | 0.000 | 67 | Load | 0.027741 | 0.026713 | 0.000 | 0.000 | 0.000 |
| 9 | Load | 0.08756 | 0.051073 | 0.000 | 0.000 | 0.000 | 68 | Load | 0.052814 | 0.025257 | 0.000 | 0.000 | 0.000 |
| 10 | Load, PV | 0.1982 | 0.10677 | 0.300 | 0.000 | 0.300 | 69 | Load | 0.06689 | 0.038713 | 0.000 | 0.000 | 0.000 |
| 11 | Load | 0.1468 | 0.075995 | 0.000 | 0.000 | 0.000 | 70 | Load | 0.4675 | 0.39514 | 0.000 | 0.000 | 0.000 |
| 12 | Load | 0.02604 | 0.018687 | 0.000 | 0.000 | 0.000 | 71 | Load, MT | 0.59485 | 0.23974 | 1.000 | 0.500 | 1.118 |
| 13 | Load | 0.0521 | 0.02322 | 0.000 | 0.000 | 0.000 | 72 | Load | 0.1325 | 0.084363 | 0.000 | 0.000 | 0.000 |
| 14 | Load | 0.1419 | 0.1175 | 0.000 | 0.000 | 0.000 | 73 | Load | 0.052699 | 0.022482 | 0.000 | 0.000 | 0.000 |
| 15 | Load | 0.02187 | 0.02879 | 0.000 | 0.000 | 0.000 | 74 | Load, PV | 0.86979 | 0.614775 | 0.600 | 0.000 | 0.600 |
| 16 | Load | 0.03337 | 0.02645 | 0.000 | 0.000 | 0.000 | 75 | Load | 0.031349 | 0.029817 | 0.000 | 0.000 | 0.000 |
| 17 | Load | 0.03243 | 0.02523 | 0.000 | 0.000 | 0.000 | 76 | Load | 0.19239 | 0.12243 | 0.000 | 0.000 | 0.000 |
| 18 | Load | 0.020234 | 0.011906 | 0.000 | 0.000 | 0.000 | 77 | Load | 0.06575 | 0.04537 | 0.000 | 0.000 | 0.000 |
| 19 | Load | 0.15694 | 0.078523 | 0.000 | 0.000 | 0.000 | 78 | Load | 0.23815 | 0.22322 | 0.000 | 0.000 | 0.000 |
| 20 | Load, MT | 0.54629 | 0.3514 | 1.000 | 0.500 | 1.118 | 79 | Load | 0.29455 | 0.16247 | 0.000 | 0.000 | 0.000 |
| 21 | Load, PV | 0.18031 | 0.1642 | 0.350 | 0.000 | 0.350 | 80 | Load | 0.48557 | 0.43792 | 0.000 | 0.000 | 0.000 |
| 22 | Load | 0.093167 | 0.054594 | 0.000 | 0.000 | 0.000 | 81 | Load, PV | 0.24353 | 0.18303 | 0.350 | 0.000 | 0.350 |
| 23 | Load | 0.08518 | 0.03965 | 0.000 | 0.000 | 0.000 | 82 | Load | 0.24353 | 0.18303 | 0.000 | 0.000 | 0.000 |
| 24 | Load, PV | 0.1681 | 0.095178 | 0.400 | 0.000 | 0.400 | 83 | Load | 0.13425 | 0.11929 | 0.000 | 0.000 | 0.000 |
| 25 | Load | 0.12511 | 0.15022 | 0.000 | 0.000 | 0.000 | 84 | Load | 0.02271 | 0.02796 | 0.000 | 0.000 | 0.000 |
| 26 | Load | 0.01603 | 0.02462 | 0.000 | 0.000 | 0.000 | 85 | Load | 0.049513 | 0.026515 | 0.000 | 0.000 | 0.000 |
| 27 | Load | 0.02603 | 0.02462 | 0.000 | 0.000 | 0.000 | 86 | Load | 0.38378 | 0.25716 | 0.000 | 0.000 | 0.000 |
| 28 | Load, MT | 0.59456 | 0.52262 | 1.000 | 0.500 | 1.118 | 87 | Load | 0.04964 | 0.0206 | 0.000 | 0.000 | 0.000 |
| 29 | Load | 0.12062 | 0.059117 | 0.000 | 0.000 | 0.000 | 88 | Load | 0.022473 | 0.011806 | 0.000 | 0.000 | 0.000 |
| 30 | Load | 0.10238 | 0.099554 | 0.000 | 0.000 | 0.000 | 89 | Load | 0.06293 | 0.04296 | 0.000 | 0.000 | 0.000 |
| 31 | Load | 0.5134 | 0.3185 | 0.000 | 0.000 | 0.000 | 90 | Load | 0.03067 | 0.03493 | 0.000 | 0.000 | 0.000 |
| 32 | Load | 0.47525 | 0.45614 | 0.000 | 0.000 | 0.000 | 91 | Load | 0.06253 | 0.06679 | 0.000 | 0.000 | 0.000 |
| 33 | Load | 0.15143 | 0.13679 | 0.000 | 0.000 | 0.000 | 92 | Load | 0.11457 | 0.081748 | 0.000 | 0.000 | 0.000 |
| 34 | Load | 0.20538 | 0.083302 | 0.000 | 0.000 | 0.000 | 93 | Load | 0.081292 | 0.066526 | 0.000 | 0.000 | 0.000 |
| 35 | Load | 0.1316 | 0.093082 | 0.000 | 0.000 | 0.000 | 94 | Load | 0.031733 | 0.01596 | 0.000 | 0.000 | 0.000 |
| 36 | Load, PV | 0.4484 | 0.36979 | 0.500 | 0.000 | 0.500 | 95 | Load | 0.03332 | 0.06048 | 0.000 | 0.000 | 0.000 |
| 37 | Load | 0.44052 | 0.32164 | 0.000 | 0.000 | 0.000 | 96 | Load, MT | 0.53128 | 0.22485 | 1.000 | 0.500 | 1.118 |
| 38 | Load | 0.11254 | 0.055134 | 0.000 | 0.000 | 0.000 | 97 | Load | 0.50703 | 0.36742 | 0.000 | 0.000 | 0.000 |
| 39 | Load | 0.053963 | 0.038998 | 0.000 | 0.000 | 0.000 | 98 | Load | 0.02639 | 0.0117 | 0.000 | 0.000 | 0.000 |
| 40 | Load | 0.39305 | 0.3426 | 0.000 | 0.000 | 0.000 | 99 | Load | 0.04599 | 0.030392 | 0.000 | 0.000 | 0.000 |
| 41 | Load | 0.32674 | 0.27856 | 0.000 | 0.000 | 0.000 | 100 | Load | 0.10066 | 0.047572 | 0.000 | 0.000 | 0.000 |
| 42 | Load, PV | 0.53626 | 0.24024 | 0.450 | 0.000 | 0.450 | 101 | Load | 0.45648 | 0.3503 | 0.000 | 0.000 | 0.000 |
| 43 | Load | 0.076247 | 0.066562 | 0.000 | 0.000 | 0.000 | 102 | Load, PV | 0.52256 | 0.44929 | 0.450 | 0.000 | 0.450 |
| 44 | Load | 0.05352 | 0.03976 | 0.000 | 0.000 | 0.000 | 103 | Load | 0.40843 | 0.16846 | 0.000 | 0.000 | 0.000 |
| 45 | Load | 0.040328 | 0.031964 | 0.000 | 0.000 | 0.000 | 104 | Load | 0.14148 | 0.13425 | 0.000 | 0.000 | 0.000 |
| 46 | Load | 0.039653 | 0.020758 | 0.000 | 0.000 | 0.000 | 105 | Load | 0.10443 | 0.066024 | 0.000 | 0.000 | 0.000 |
| 47 | Load | 0.066195 | 0.042361 | 0.000 | 0.000 | 0.000 | 106 | Load | 0.096793 | 0.083647 | 0.000 | 0.000 | 0.000 |
| 48 | Load | 0.073904 | 0.051653 | 0.000 | 0.000 | 0.000 | 107 | Load | 0.49392 | 0.41934 | 0.000 | 0.000 | 0.000 |
| 49 | Load | 0.11477 | 0.057965 | 0.000 | 0.000 | 0.000 | 108 | Load, MT | 0.22538 | 0.13588 | 1.000 | 0.500 | 1.118 |
| 50 | Load, PV | 0.91837 | 1.2051 | 0.650 | 0.000 | 0.650 | 109 | Load | 0.50921 | 0.38721 | 0.000 | 0.000 | 0.000 |
| 51 | Load, MT | 0.2103 | 0.14666 | 1.000 | 0.500 | 1.118 | 110 | Load | 0.1885 | 0.17346 | 0.000 | 0.000 | 0.000 |
| 52 | Load | 0.06668 | 0.056608 | 0.000 | 0.000 | 0.000 | 111 | Load, PV | 0.91803 | 0.89855 | 0.600 | 0.000 | 0.600 |
| 53 | Load | 0.042207 | 0.040184 | 0.000 | 0.000 | 0.000 | 112 | Load | 0.30508 | 0.21537 | 0.000 | 0.000 | 0.000 |
| 54 | Load | 0.43374 | 0.28341 | 0.000 | 0.000 | 0.000 | 113 | Load | 0.05438 | 0.04097 | 0.000 | 0.000 | 0.000 |
| 55 | Load | 0.0621 | 0.02686 | 0.000 | 0.000 | 0.000 | 114 | Load | 0.21114 | 0.1929 | 0.000 | 0.000 | 0.000 |
| 56 | Load | 0.09246 | 0.08838 | 0.000 | 0.000 | 0.000 | 115 | Load | 0.067009 | 0.053336 | 0.000 | 0.000 | 0.000 |
| 57 | Load | 0.085188 | 0.055436 | 0.000 | 0.000 | 0.000 | 116 | Load | 0.16207 | 0.090321 | 0.000 | 0.000 | 0.000 |
| 58 | Load | 0.3453 | 0.3324 | 0.000 | 0.000 | 0.000 | 117 | Load | 0.048785 | 0.029156 | 0.000 | 0.000 | 0.000 |
| 59 | Load | 0.0225 | 0.01683 | 0.000 | 0.000 | 0.000 | 118 | Load | 0.0339 | 0.01898 | 0.000 | 0.000 | 0.000 |

Table 10. CN communication link parameters

| Link | From | To | Type | Capacity | Delay | Link | From | To | Type | Capacity | Delay | Link | From | To | Type | Capacity | Delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 | Server | RT1 | Wired | 1000 | 1 | C65 | SW5 | FTU67 | Wired | 50 | 1 | C129 | SW10 | FTU14 | Wired | 50 | 1 |
| C2 | Server | RT2 | Wired | 1000 | 1 | C66 | SW5 | FTU68 | Wired | 50 | 1 | C130 | SW10 | FTU18 | Wired | 50 | 1 |
| C3 | RT1 | RT2 | Wired | 800 | 1 | C67 | SW5 | FTU69 | Wired | 50 | 1 | C131 | SW10 | FTU19 | Wired | 50 | 1 |
| C4 | RT1 | SW1 | Wired | 200 | 1 | C68 | SW5 | FTU70 | Wired | 50 | 1 | C132 | SW11 | FTU7 | Wired | 50 | 1 |
| C5 | RT2 | SW10 | Wired | 200 | 1 | C69 | SW5 | FTU91 | Wired | 50 | 1 | C133 | SW11 | FTU8 | Wired | 50 | 1 |
| C6 | SW1 | SW2 | Wired | 150 | 1 | C70 | SW5 | FTU92 | Wired | 50 | 1 | C134 | SW11 | FTU15 | Wired | 50 | 1 |
| C7 | SW2 | SW3 | Wired | 150 | 1 | C71 | SW5 | FTU93 | Wired | 50 | 1 | C135 | SW11 | FTU16 | Wired | 50 | 1 |
| C8 | SW1 | SW4 | Wired | 150 | 1 | C72 | SW5 | FTU94 | Wired | 50 | 1 | C136 | SW11 | FTU17 | Wired | 50 | 1 |
| C9 | SW2 | SW5 | Wired | 150 | 1 | C73 | SW5 | FTU96 | Wired | 50 | 1 | C137 | SW11 | FTU20 | Wired | 50 | 1 |
| C10 | SW3 | SW6 | Wired | 150 | 1 | C74 | SW5 | FTU97 | Wired | 50 | 1 | C138 | SW11 | FTU21 | Wired | 50 | 1 |
| C11 | SW4 | SW7 | Wired | 150 | 1 | C75 | SW5 | FTU98 | Wired | 50 | 1 | C139 | SW11 | FTU22 | Wired | 50 | 1 |
| C12 | SW5 | SW8 | Wired | 150 | 1 | C76 | SW5 | FTU99 | Wired | 50 | 1 | C140 | SW11 | FTU121 | Wired | 50 | 1 |
| C13 | SW6 | SW9 | Wired | 150 | 1 | C77 | SW5 | FTU123 | Wired | 50 | 1 | C141 | SW12 | FTU23 | Wired | 50 | 1 |
| C14 | SW7 | SW10 | Wired | 150 | 1 | C78 | SW6 | FTU71 | Wired | 50 | 1 | C142 | SW12 | FTU24 | Wired | 50 | 1 |
| C15 | SW8 | SW11 | Wired | 150 | 1 | C79 | SW6 | FTU72 | Wired | 50 | 1 | C143 | SW12 | FTU25 | Wired | 50 | 1 |
| C16 | SW9 | SW12 | Wired | 150 | 1 | C80 | SW6 | FTU73 | Wired | 50 | 1 | C144 | SW12 | FTU26 | Wired | 50 | 1 |
| C17 | SW10 | SW11 | Wired | 150 | 1 | C81 | SW6 | FTU74 | Wired | 50 | 1 | C145 | SW12 | FTU27 | Wired | 50 | 1 |
| C18 | SW11 | SW12 | Wired | 150 | 1 | C82 | SW6 | FTU75 | Wired | 50 | 1 | C146 | SW12 | FTU44 | Wired | 50 | 1 |
| C19 | SW1 | FTU78 | Wired | 50 | 1 | C83 | SW6 | FTU76 | Wired | 50 | 1 | C147 | SW12 | FTU45 | Wired | 50 | 1 |
| C20 | SW1 | FTU79 | Wired | 50 | 1 | C84 | SW6 | FTU77 | Wired | 50 | 1 | C148 | SW12 | FTU46 | Wired | 50 | 1 |
| C21 | SW1 | FTU80 | Wired | 50 | 1 | C85 | SW6 | FTU95 | Wired | 50 | 1 | C149 | SW12 | FTU119 | Wired | 50 | 1 |
| C22 | SW1 | FTU86 | Wired | 50 | 1 | C86 | SW6 | FTU127 | Wired | 50 | 1 | C150 | SW12 | FTU120 | Wired | 50 | 1 |
| C23 | SW1 | FTU87 | Wired | 50 | 1 | C87 | SW6 | FTU129 | Wired | 50 | 1 | C151 | Server | BS1 | 5G | 500 | 5 |
| C24 | SW1 | FTU100 | Wired | 50 | 1 | C88 | SW7 | FTU3 | Wired | 50 | 1 | C152 | Server | BS2 | 5G | 500 | 5 |
| C25 | SW1 | FTU101 | Wired | 50 | 1 | C89 | SW7 | FTU28 | Wired | 50 | 1 | C153 | Server | BS3 | 5G | 500 | 5 |
| C26 | SW1 | FTU102 | Wired | 50 | 1 | C90 | SW7 | FTU29 | Wired | 50 | 1 | C154 | Server | BS4 | 5G | 500 | 5 |
| C27 | SW1 | FTU103 | Wired | 50 | 1 | C91 | SW7 | FTU30 | Wired | 50 | 1 | C155 | Server | BS5 | 5G | 500 | 5 |
| C28 | SW1 | FTU112 | Wired | 50 | 1 | C92 | SW7 | FTU31 | Wired | 50 | 1 | C156 | Server | BS6 | 5G | 500 | 5 |
| C29 | SW1 | FTU113 | Wired | 50 | 1 | C93 | SW7 | FTU32 | Wired | 50 | 1 | C157 | BS1 | FTU107 | 5G | 200 | 5 |
| C30 | SW1 | FTU114 | Wired | 50 | 1 | C94 | SW7 | FTU36 | Wired | 50 | 1 | C158 | BS1 | FTU108 | 5G | 200 | 5 |
| C31 | SW1 | FTU115 | Wired | 50 | 1 | C95 | SW7 | FTU37 | Wired | 50 | 1 | C159 | BS1 | FTU109 | 5G | 200 | 5 |
| C32 | SW2 | FTU81 | Wired | 50 | 1 | C96 | SW7 | FTU38 | Wired | 50 | 1 | C160 | BS1 | FTU110 | 5G | 200 | 5 |
| C33 | SW2 | FTU82 | Wired | 50 | 1 | C97 | SW7 | FTU39 | Wired | 50 | 1 | C161 | BS1 | FTU132 | 5G | 200 | 5 |
| C34 | SW2 | FTU83 | Wired | 50 | 1 | C98 | SW7 | FTU55 | Wired | 50 | 1 | C162 | BS2 | FTU80 | 5G | 200 | 5 |
| C35 | SW2 | FTU88 | Wired | 50 | 1 | C99 | SW7 | FTU56 | Wired | 50 | 1 | C163 | BS2 | FTU81 | 5G | 200 | 5 |
| C36 | SW2 | FTU104 | Wired | 50 | 1 | C100 | SW7 | FTU57 | Wired | 50 | 1 | C164 | BS2 | FTU86 | 5G | 200 | 5 |
| C37 | SW2 | FTU105 | Wired | 50 | 1 | C101 | SW7 | FTU124 | Wired | 50 | 1 | C165 | BS2 | FTU87 | 5G | 200 | 5 |
| C38 | SW2 | FTU106 | Wired | 50 | 1 | C102 | SW8 | FTU9 | Wired | 50 | 1 | C166 | BS2 | FTU88 | 5G | 200 | 5 |
| C39 | SW2 | FTU116 | Wired | 50 | 1 | C103 | SW8 | FTU33 | Wired | 50 | 1 | C167 | BS2 | FTU131 | 5G | 200 | 5 |
| C40 | SW2 | FTU117 | Wired | 50 | 1 | C104 | SW8 | FTU34 | Wired | 50 | 1 | C168 | BS3 | FTU37 | 5G | 200 | 5 |
| C41 | SW2 | FTU118 | Wired | 50 | 1 | C105 | SW8 | FTU35 | Wired | 50 | 1 | C169 | BS3 | FTU59 | 5G | 200 | 5 |
| C42 | SW2 | FTU131 | Wired | 50 | 1 | C106 | SW8 | FTU40 | Wired | 50 | 1 | C170 | BS3 | FTU60 | 5G | 200 | 5 |
| C43 | SW3 | FTU84 | Wired | 50 | 1 | C107 | SW8 | FTU41 | Wired | 50 | 1 | C171 | BS3 | FTU61 | 5G | 200 | 5 |
| C44 | SW3 | FTU85 | Wired | 50 | 1 | C108 | SW8 | FTU42 | Wired | 50 | 1 | C172 | BS3 | FTU62 | 5G | 200 | 5 |
| C45 | SW3 | FTU107 | Wired | 50 | 1 | C109 | SW8 | FTU47 | Wired | 50 | 1 | C173 | BS3 | FTU124 | 5G | 200 | 5 |
| C46 | SW3 | FTU108 | Wired | 50 | 1 | C110 | SW8 | FTU125 | Wired | 50 | 1 | C174 | BS3 | FTU126 | 5G | 200 | 5 |
| C47 | SW3 | FTU109 | Wired | 50 | 1 | C111 | SW9 | FTU43 | Wired | 50 | 1 | C175 | BS4 | FTU51 | 5G | 200 | 5 |
| C48 | SW3 | FTU110 | Wired | 50 | 1 | C112 | SW9 | FTU48 | Wired | 50 | 1 | C176 | BS4 | FTU52 | 5G | 200 | 5 |
| C49 | SW3 | FTU111 | Wired | 50 | 1 | C113 | SW9 | FTU49 | Wired | 50 | 1 | C177 | BS4 | FTU53 | 5G | 200 | 5 |
| C50 | SW3 | FTU128 | Wired | 50 | 1 | C114 | SW9 | FTU50 | Wired | 50 | 1 | C178 | BS4 | FTU129 | 5G | 200 | 5 |
| C51 | SW3 | FTU130 | Wired | 50 | 1 | C115 | SW9 | FTU51 | Wired | 50 | 1 | C179 | BS5 | FTU2 | 5G | 200 | 5 |
| C52 | SW3 | FTU132 | Wired | 50 | 1 | C116 | SW9 | FTU52 | Wired | 50 | 1 | C180 | BS5 | FTU3 | 5G | 200 | 5 |
| C53 | SW4 | FTU58 | Wired | 50 | 1 | C117 | SW9 | FTU53 | Wired | 50 | 1 | C181 | BS5 | FTU4 | 5G | 200 | 5 |
| C54 | SW4 | FTU59 | Wired | 50 | 1 | C118 | SW9 | FTU54 | Wired | 50 | 1 | C182 | BS5 | FTU28 | 5G | 200 | 5 |
| C55 | SW4 | FTU60 | Wired | 50 | 1 | C119 | SW9 | FTU122 | Wired | 50 | 1 | C183 | BS5 | FTU29 | 5G | 200 | 5 |
| C56 | SW4 | FTU61 | Wired | 50 | 1 | C120 | SW9 | FTU133 | Wired | 50 | 1 | C184 | BS5 | FTU55 | 5G | 200 | 5 |
| C57 | SW4 | FTU63 | Wired | 50 | 1 | C121 | SW10 | FTU2 | Wired | 50 | 1 | C185 | BS6 | FTU9 | 5G | 200 | 5 |
| C58 | SW4 | FTU64 | Wired | 50 | 1 | C122 | SW10 | FTU4 | Wired | 50 | 1 | C186 | BS6 | FTU42 | 5G | 200 | 5 |
| C59 | SW4 | FTU65 | Wired | 50 | 1 | C123 | SW10 | FTU5 | Wired | 50 | 1 | C187 | BS6 | FTU43 | 5G | 200 | 5 |
| C60 | SW4 | FTU66 | Wired | 50 | 1 | C124 | SW10 | FTU6 | Wired | 50 | 1 | C188 | BS6 | FTU44 | 5G | 200 | 5 |
| C61 | SW4 | FTU89 | Wired | 50 | 1 | C125 | SW10 | FTU10 | Wired | 50 | 1 | C189 | BS6 | FTU45 | 5G | 200 | 5 |
| C62 | SW4 | FTU90 | Wired | 50 | 1 | C126 | SW10 | FTU11 | Wired | 50 | 1 | C190 | BS6 | FTU121 | 5G | 200 | 5 |
| C63 | SW4 | FTU126 | Wired | 50 | 1 | C127 | SW10 | FTU12 | Wired | 50 | 1 | - | - | - | - | - | - |
| C64 | SW5 | FTU62 | Wired | 50 | 1 | C128 | SW10 | FTU13 | Wired | 50 | 1 | - | - | - | - | - | - |

Table 11. TN road parameters

| Road | From | To | Capacity/(veh·h⁻¹) | Length/m | Free-flow time/h | Road | From | To | Capacity/(veh·h⁻¹) | Length/m | Free-flow time/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | T2 | T1 | 1200 | 1860 | 0.1302 | R67 | T37 | T43 | 200 | 310 | 0.0217 |
| R2 | T2 | T9 | 500 | 790 | 0.0553 | R68 | T38 | T36 | 1300 | 1010 | 0.0707 |
| R3 | T3 | T2 | 500 | 770 | 0.0539 | R69 | T38 | T46 | 460 | 480 | 0.0336 |
| R4 | T3 | T6 | 600 | 620 | 0.0434 | R70 | T39 | T38 | 580 | 900 | 0.063 |
| R5 | T4 | T5 | 700 | 720 | 0.0504 | R71 | T39 | T51 | 420 | 670 | 0.0469 |
| R6 | T4 | T10 | 720 | 740 | 0.0518 | R72 | T40 | T39 | 340 | 530 | 0.0371 |
| R7 | T5 | T12 | 820 | 850 | 0.0595 | R73 | T40 | T57 | 300 | 950 | 0.0665 |
| R8 | T6 | T9 | 840 | 880 | 0.0616 | R74 | T41 | T40 | 320 | 500 | 0.035 |
| R9 | T6 | T19 | 940 | 980 | 0.0686 | R75 | T41 | T59 | 1180 | 1230 | 0.0861 |
| R10 | T7 | T10 | 620 | 640 | 0.0448 | R76 | T42 | T54 | 340 | 540 | 0.0378 |
| R11 | T7 | T15 | 480 | 750 | 0.0525 | R77 | T43 | T42 | 1240 | 1290 | 0.0903 |
| R12 | T8 | T7 | 980 | 1020 | 0.0714 | R78 | T43 | T55 | 260 | 420 | 0.0294 |
| R13 | T8 | T16 | 720 | 740 | 0.0518 | R79 | T44 | T43 | 900 | 940 | 0.0658 |
| R14 | T9 | T8 | 1160 | 1210 | 0.0847 | R80 | T44 | T52 | 350 | 310 | 0.0217 |
| R15 | T9 | T17 | 500 | 790 | 0.0553 | R81 | T45 | T44 | 350 | 270 | 0.0189 |
| R16 | T10 | T5 | 620 | 640 | 0.0448 | R82 | T45 | T50 | 240 | 240 | 0.0168 |
| R17 | T10 | T14 | 520 | 550 | 0.0385 | R83 | T46 | T48 | 980 | 1020 | 0.0714 |
| R18 | T11 | T20 | 660 | 690 | 0.0483 | R84 | T47 | T45 | 860 | 900 | 0.063 |
| R19 | T12 | T11 | 420 | 430 | 0.0301 | R85 | T47 | T58 | 360 | 560 | 0.0392 |
| R20 | T12 | T21 | 640 | 660 | 0.0462 | R86 | T48 | T49 | 600 | 630 | 0.0441 |
| R21 | T13 | T12 | 320 | 340 | 0.0238 | R87 | T49 | T47 | 900 | 930 | 0.0651 |
| R22 | T13 | T22 | 380 | 600 | 0.042 | R88 | T49 | T63 | 500 | 790 | 0.0553 |
| R23 | T14 | T13 | 640 | 660 | 0.0462 | R89 | T50 | T52 | 220 | 350 | 0.0245 |
| R24 | T14 | T23 | 580 | 610 | 0.0427 | R90 | T50 | T66 | 880 | 920 | 0.0644 |
| R25 | T15 | T14 | 600 | 620 | 0.0434 | R91 | T51 | T46 | 740 | 780 | 0.0546 |
| R26 | T15 | T24 | 400 | 620 | 0.0434 | R92 | T52 | T53 | 320 | 490 | 0.0343 |
| R27 | T16 | T15 | 960 | 1010 | 0.0707 | R93 | T52 | T65 | 280 | 850 | 0.0595 |
| R28 | T16 | T26 | 580 | 610 | 0.0427 | R94 | T53 | T55 | 220 | 350 | 0.0245 |
| R29 | T17 | T16 | 1100 | 1140 | 0.0798 | R95 | T53 | T64 | 480 | 740 | 0.0518 |
| R30 | T17 | T27 | 380 | 600 | 0.042 | R96 | T54 | T62 | 420 | 660 | 0.0462 |
| R31 | T18 | T17 | 440 | 450 | 0.0315 | R97 | T55 | T56 | 260 | 410 | 0.0287 |
| R32 | T18 | T28 | 200 | 580 | 0.0406 | R98 | T55 | T60 | 420 | 670 | 0.0469 |
| R33 | T19 | T18 | 460 | 470 | 0.0329 | R99 | T56 | T54 | 600 | 940 | 0.0658 |
| R34 | T19 | T29 | 540 | 570 | 0.0399 | R100 | T56 | T61 | 560 | 590 | 0.0413 |
| R35 | T20 | T32 | 780 | 810 | 0.0567 | R101 | T57 | T51 | 660 | 680 | 0.0476 |
| R36 | T21 | T20 | 400 | 410 | 0.0287 | R102 | T58 | T50 | 560 | 880 | 0.0616 |
| R37 | T21 | T33 | 720 | 750 | 0.0525 | R103 | T58 | T67 | 480 | 760 | 0.0532 |
| R38 | T22 | T21 | 840 | 660 | 0.0462 | R104 | T59 | T57 | 540 | 570 | 0.0399 |
| R39 | T22 | T34 | 440 | 690 | 0.0483 | R105 | T60 | T61 | 200 | 230 | 0.0161 |
| R40 | T23 | T22 | 760 | 590 | 0.0413 | R106 | T60 | T72 | 480 | 750 | 0.0525 |
| R41 | T23 | T35 | 660 | 690 | 0.0483 | R107 | T61 | T62 | 620 | 960 | 0.0672 |
| R42 | T24 | T23 | 800 | 620 | 0.0434 | R108 | T61 | T74 | 720 | 760 | 0.0532 |
| R43 | T24 | T36 | 440 | 690 | 0.0483 | R109 | T62 | T73 | 480 | 760 | 0.0532 |
| R44 | T25 | T31 | 280 | 430 | 0.0301 | R110 | T63 | T58 | 700 | 1080 | 0.0756 |
| R45 | T26 | T24 | 1300 | 1010 | 0.0707 | R111 | T64 | T60 | 340 | 540 | 0.0378 |
| R46 | T26 | T38 | 660 | 690 | 0.0483 | R112 | T64 | T68 | 400 | 640 | 0.0448 |
| R47 | T27 | T26 | 660 | 1030 | 0.0721 | R113 | T65 | T64 | 200 | 270 | 0.0189 |
| R48 | T27 | T39 | 440 | 690 | 0.0483 | R114 | T65 | T69 | 200 | 600 | 0.042 |
| R49 | T28 | T27 | 280 | 440 | 0.0308 | R115 | T66 | T65 | 240 | 370 | 0.0259 |
| R50 | T28 | T40 | 220 | 680 | 0.0476 | R116 | T66 | T70 | 540 | 560 | 0.0392 |
| R51 | T29 | T28 | 340 | 530 | 0.0371 | R117 | T67 | T66 | 340 | 540 | 0.0378 |
| R52 | T29 | T41 | 660 | 680 | 0.0476 | R118 | T68 | T71 | 420 | 440 | 0.0308 |
| R53 | T30 | T25 | 1120 | 1170 | 0.0819 | R119 | T68 | T77 | 320 | 510 | 0.0357 |
| R54 | T30 | T37 | 360 | 550 | 0.0385 | R120 | T69 | T68 | 300 | 310 | 0.0217 |
| R55 | T31 | T42 | 300 | 470 | 0.0329 | R121 | T70 | T69 | 220 | 220 | 0.0154 |
| R56 | T32 | T30 | 1060 | 1110 | 0.0777 | R122 | T71 | T72 | 200 | 140 | 0.0098 |
| R57 | T32 | T44 | 680 | 710 | 0.0497 | R123 | T71 | T76 | 480 | 490 | 0.0343 |
| R58 | T33 | T32 | 360 | 370 | 0.0259 | R124 | T72 | T74 | 200 | 200 | 0.014 |
| R59 | T33 | T45 | 660 | 690 | 0.0483 | R125 | T74 | T73 | 900 | 930 | 0.0651 |
| R60 | T34 | T33 | 1280 | 1000 | 0.07 | R126 | T74 | T75 | 420 | 440 | 0.0308 |
| R61 | T34 | T47 | 500 | 770 | 0.0539 | R127 | T76 | T75 | 300 | 460 | 0.0322 |
| R62 | T35 | T34 | 700 | 550 | 0.0385 | R128 | T76 | T78 | 420 | 430 | 0.0301 |
| R63 | T35 | T49 | 680 | 700 | 0.049 | R129 | T77 | T76 | 200 | 310 | 0.0217 |
| R64 | T36 | T35 | 800 | 620 | 0.0434 | R130 | T1 | T79 | 1240 | 1300 | 0.091 |
| R65 | T36 | T48 | 380 | 600 | 0.042 | R131 | T79 | T4 | 620 | 640 | 0.0448 |
| R66 | T37 | T31 | 400 | 1230 | 0.0861 | R132 | T79 | T8 | 1140 | 1790 | 0.1253 |

Table 12. Origin-destination pairs and candidate paths

| O-D pair | O | D | Trip type | O-D pair | O | D | Trip type | O-D pair | O | D | Trip type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T3 | T6 | To university campus | 21 | T3 | T2 | To leisure or shopping venue | 41 | T36 | T25 | From work |
| 2 | T3 | T59 | To work | 22 | T15 | T2 | To leisure or shopping venue | 42 | T64 | T31 | From work |
| 3 | T79 | T53 | To work | 23 | T51 | T63 | To leisure or shopping venue | 43 | T72 | T31 | From work in medical district |
| 4 | T4 | T30 | To work | 24 | T25 | T30 | To leisure or shopping venue | 44 | T48 | T27 | From university campus |
| 5 | T15 | T6 | To university campus | 25 | T31 | T53 | To leisure or shopping venue | 45 | T36 | T29 | From work |
| 6 | T15 | T36 | To work | 26 | T31 | T77 | To leisure or shopping venue | 46 | T38 | T3 | From work |
| 7 | T27 | T59 | To work | 27 | T4 | T52 | To leisure or shopping venue | 47 | T65 | T79 | From work |
| 8 | T29 | T53 | To work | 28 | T27 | T65 | To leisure or shopping venue | 48 | T53 | T15 | From work |
| 9 | T51 | T38 | To work | 29 | T36 | T77 | To leisure or shopping venue | 49 | T65 | T25 | From work |
| 10 | T51 | T53 | To work | 30 | T29 | T72 | To medical visit | 50 | T59 | T51 | From work |
| 11 | T25 | T36 | To work | 31 | T6 | T3 | From university campus | 51 | T2 | T3 | From leisure or shopping venue |
| 12 | T31 | T64 | To work | 32 | T59 | T3 | From work | 52 | T2 | T15 | From leisure or shopping venue |
| 13 | T31 | T72 | To work in medical district | 33 | T53 | T79 | From work | 53 | T63 | T51 | From leisure or shopping venue |
| 14 | T27 | T48 | To university campus | 34 | T30 | T4 | From work | 54 | T30 | T25 | From leisure or shopping venue |
| 15 | T29 | T36 | To work | 35 | T6 | T15 | From university campus | 55 | T53 | T31 | From leisure or shopping venue |
| 16 | T3 | T38 | To work | 36 | T36 | T15 | From work | 56 | T77 | T31 | From leisure or shopping venue |
| 17 | T79 | T65 | To work | 37 | T59 | T27 | From work | 57 | T52 | T4 | From leisure or shopping venue |
| 18 | T15 | T53 | To work | 38 | T53 | T29 | From work | 58 | T65 | T27 | From leisure or shopping venue |
| 19 | T25 | T65 | To work | 39 | T38 | T51 | From work | 59 | T77 | T36 | From leisure or shopping venue |
| 20 | T51 | T59 | To work | 40 | T53 | T51 | From work | 60 | T72 | T29 | From medical visit |

Table 13. Hourly origin-destination travel demand.

| OD pair Period | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 78 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 22 | 44 | 108 | 96 | 44 | 10 | 2 | 8 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | 0 | 0 | 0 | 24 | 46 | 116 | 104 | 46 | 12 | 2 | 8 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 20 | 38 | 92 | 82 | 38 | 10 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 78 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 | 0 | 20 | 38 | 92 | 82 | 38 | 10 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 78 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0 | 0 | 0 | 0 | 18 | 34 | 84 | 76 | 34 | 8 | 2 | 6 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 0 | 0 | 0 | 20 | 38 | 92 | 82 | 38 | 10 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 0 | 0 | 0 | 0 | 22 | 40 | 100 | 90 | 40 | 10 | 2 | 6 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 78 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 12 | 0 | 0 | 0 | 0 | 0 | 18 | 34 | 84 | 76 | 34 | 8 | 2 | 6 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 13 | 0 | 0 | 0 | 0 | 0 | 14 | 28 | 68 | 62 | 28 | 8 | 2 | 4 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 14 | 0 | 0 | 0 | 0 | 0 | 14 | 28 | 68 | 62 | 28 | 8 | 2 | 4 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 15 | 0 | 0 | 0 | 0 | 0 | 18 | 34 | 84 | 76 | 34 | 8 | 2 | 6 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 16 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 78 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 17 | 0 | 0 | 0 | 0 | 0 | 20 | 38 | 92 | 82 | 38 | 10 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 18 | 0 | 0 | 0 | 0 | 0 | 22 | 40 | 100 | 90 | 40 | 10 | 2 | 6 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 19 | 0 | 0 | 0 | 0 | 0 | 16 | 32 | 76 | 68 | 30 | 8 | 2 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 20 | 0 | 0 | 0 | 0 | 0 | 12 | 24 | 62 | 56 | 24 | 6 | 2 | 4 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 |
| 21 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 2 | 18 | 26 | 30 | 12 | 8 | 10 | 14 | 24 | 14 | 12 | 6 | 2 | 0 | 0 | 0 |
| 22 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 2 | 18 | 26 | 30 | 12 | 8 | 10 | 14 | 24 | 14 | 12 | 6 | 2 | 0 | 0 | 0 |
| 23 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 16 | 24 | 28 | 12 | 8 | 10 | 12 | 22 | 12 | 10 | 4 | 2 | 0 | 0 | 0 |
| 24 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 2 | 18 | 26 | 30 | 12 | 8 | 10 | 14 | 24 | 14 | 12 | 6 | 2 | 0 | 0 | 0 |
| 25 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 2 | 18 | 26 | 30 | 12 | 8 | 10 | 14 | 24 | 14 | 12 | 6 | 2 | 0 | 0 | 0 |
| 26 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 4 | 22 | 32 | 36 | 14 | 10 | 12 | 16 | 28 | 18 | 14 | 6 | 2 | 0 | 0 | 0 |
| 27 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 4 | 20 | 30 | 34 | 14 | 8 | 12 | 14 | 26 | 16 | 12 | 6 | 0 | 0 | 0 | 0 |
| 28 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 14 | 22 | 24 | 10 | 6 | 8 | 10 | 18 | 12 | 10 | 4 | 2 | 0 | 0 | 0 |
| 29 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 2 | 2 | 18 | 26 | 30 | 12 | 8 | 10 | 14 | 24 | 14 | 12 | 6 | 2 | 0 | 0 | 0 |
| 30 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 14 | 22 | 24 | 10 | 6 | 8 | 10 | 18 | 12 | 10 | 4 | 2 | 0 | 0 | 0 |
| 31 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 32 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 0 | 0 | 8 | 0 | 0 | 14 | 40 | 76 | 72 | 36 | 44 | 30 | 8 | 4 |
| 33 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 0 | 0 | 8 | 0 | 0 | 16 | 42 | 82 | 78 | 38 | 48 | 32 | 8 | 4 |
| 34 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 14 | 34 | 64 | 62 | 30 | 40 | 26 | 6 | 4 |
| 35 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 36 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 14 | 34 | 64 | 62 | 30 | 40 | 26 | 6 | 4 |
| 37 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 38 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 12 | 32 | 60 | 56 | 28 | 36 | 24 | 6 | 2 |
| 39 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 14 | 34 | 64 | 62 | 30 | 40 | 26 | 6 | 4 |
| 40 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 0 | 0 | 8 | 0 | 0 | 14 | 36 | 70 | 66 | 34 | 42 | 28 | 6 | 4 |
| 41 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 42 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 12 | 32 | 60 | 56 | 28 | 36 | 24 | 6 | 2 |
| 43 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 4 | 0 | 0 | 10 | 26 | 48 | 46 | 24 | 30 | 20 | 4 | 2 |
| 44 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 4 | 0 | 0 | 10 | 26 | 48 | 46 | 24 | 30 | 20 | 4 | 2 |
| 45 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 12 | 32 | 60 | 56 | 28 | 36 | 24 | 6 | 2 |
| 46 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 47 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 14 | 34 | 64 | 62 | 30 | 40 | 26 | 6 | 4 |
| 48 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 0 | 0 | 8 | 0 | 0 | 14 | 36 | 70 | 66 | 34 | 42 | 28 | 6 | 4 |
| 49 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 6 | 0 | 0 | 10 | 28 | 54 | 52 | 26 | 32 | 22 | 6 | 2 |
| 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 4 | 0 | 0 | 8 | 22 | 44 | 42 | 20 | 26 | 18 | 4 | 2 |
| 51 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 16 | 14 | 10 | 8 | 12 | 14 | 12 | 18 | 24 | 16 | 8 |
| 52 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 16 | 14 | 10 | 8 | 12 | 14 | 12 | 18 | 24 | 16 | 8 |
| 53 | 6 | 4 | 4 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 14 | 14 | 8 | 8 | 10 | 12 | 10 | 16 | 22 | 14 | 8 |
| 54 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 16 | 14 | 10 | 8 | 12 | 14 | 12 | 18 | 24 | 16 | 8 |
| 55 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 16 | 14 | 10 | 8 | 12 | 14 | 12 | 18 | 24 | 16 | 8 |
| 56 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 16 | 20 | 18 | 12 | 10 | 14 | 16 | 14 | 22 | 28 | 20 | 10 |
| 57 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 14 | 18 | 16 | 10 | 10 | 12 | 16 | 14 | 20 | 26 | 18 | 8 |
| 58 | 4 | 2 | 4 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 14 | 12 | 8 | 6 | 8 | 12 | 10 | 14 | 20 | 12 | 6 |
| 59 | 6 | 4 | 6 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 16 | 14 | 10 | 8 | 12 | 14 | 12 | 18 | 24 | 16 | 8 |
| 60 | 4 | 2 | 4 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 14 | 12 | 8 | 6 | 8 | 12 | 10 | 14 | 20 | 12 | 6 |

Model and Solver Parameters

| Parameter | Value |
| --- | --- |
| Base restored-load weight | 2000 |
| DN / CN / TN objective weights | 1 / 0.1 / 0.1 |
| Grid / PV / MT generation-cost coefficients | 20 / 0.1 / 30 |
| DN-isolation / CN virtual-source penalties | 500 / 5000 |
| V2G price-to-travel-cost coefficient | 0.25 |
| FTU information demand | 2 / 5 / 10 flow units |
| CN relay-flow bound | 447 flow units |
| Big-M values (DN / TN) | 500 / 1000 |
| ADMM box / sphere penalties | Initially 50; ×1.5 every five iterations, capped at 1000 |
| ADMM stopping tolerances (absolute / relative / integrality) | 10-4 / 10-3 / 10-3 |